## Auger Filler -  Pneumatic Pouch Packing Machine (HT -012)

## 1. Machine Description

- Machine Type: Auger Filler Fully Pneumatic Pouch Packing Machine

- Packaging Style: Vertical Form-Fill-Seal (VFFS)

- Operation: Fully pneumatic (film pulling, filling, sealing & cutting via pneumatic cylinders)

- Filling Principle: Auger screw-based volumetric filling

- Automation Level: Fully automatic

## 2. Applications

Designed for non-free-flowing and semi-free-flow powders, including:

- Spice powders & masalas

- Wheat flour / atta

- Milk powder

- Coffee powder

- Detergent powder

- Talcum powder

- Chemical & industrial powders

## 3. Filling System

- Filling Method: Auger screw filler

- Auger Drive: Pneumatic-assisted auger with electric motor drive

- Hopper Capacity: 25 – 50 kg

- Filling Range:

  - 5 g – 50 g

  - 50 g – 500 g

  - Optional up to 1 kg

- Filling Accuracy: ±1.5 % (product dependent)

- Level Control: Hopper level sensor (optional)

## 4. Pouch Specifications

- Pouch Type:

  - Center seal (standard)

  - Three-side seal (optional)

- Pouch Width: 50 mm – 200 mm

- Pouch Length: 50 mm – 300 mm

- Film Material: Heat Sealable

- Maximum Film Width: 450 mm

## 5. Sealing System

- Sealing Type: Fully pneumatic vertical & horizontal heat sealing

- Seal Width: 8 – 10 mm

- Temperature Control: Digital PID temperature controller

- Jaw Pressure: Pneumatically adjustable for laminated films

## 6. Performance

- Packing Speed:
20 – 50 pouches per minute
(varies with powder characteristics, fill weight & pouch size)

## 7. Control System

- Pneumatic logic control with electronic timers & solenoid valves

- Separate control for auger dosing time

- Optional photo-electric sensor for printed film registration

## 8. Power & Pneumatics

- Power Supply: 220 V AC, Single Phase, 50 Hz OR Three Face

- Power Consumption: Approx. 3 – 4 kW

- Compressed Air Requirement:
6 – 8 CFM @ 6 bar (6 kg/cm²)

- Air Quality: Dry & filtered compressed air (FRL unit mandatory)

## 9. Construction

- Machine Frame: Heavy-duty Mild Steel, powder-coated

- Product Contact Parts: SS-304 (food-grade)

- Auger Screw & Funnel: Stainless steel, removable for cleaning

- Fasteners: Stainless steel

## 10. Dimensions & Weight

- Machine Dimensions (Approx.):
1200 mm (L) × 1100 mm (W) × 2400 mm (H)

- Machine Weight: Approx. 600 – 750 kg

## 11. Safety Features

- Emergency stop button

- Guarded sealing & cutting zone

- Over-temperature protection

- Pneumatic pressure safety valves

## 12. Optional Accessories

- Servo-driven auger (for higher accuracy)

- Nitrogen flushing system

- Vacuum or dust extraction system

- Batch / date / lot coding unit

- Screw feeder for powder feeding

- Output conveyor

## 13. Utilities to Be Provided by Buyer

- Single-phase electrical connection

- Compressed air supply with dryer & FRL unit

- Clean, dry & level working environment

### ✅ Key Advantages

- Suitable for difficult-to-handle powders

- Reliable pneumatic sealing pressure

- Consistent pouch quality

- Simple operation & maintenance

- Cost-effective alternative to servo-driven systems