Automatic fully pneumatic cup filler packing machine ( HT - 011)

## Technical Specifications —

## Fully Pneumatic Cup Filler Pouch Packing Machine

### 🔧 Filling System

- Type: Volumetric cup filler with interchangeable cups for different weights.

- Filling Mechanism: Semi-automatic — pneumatic assist for pouch feed/closing and manual/servo cup actuation.

- Accuracy: Approximately ±0.2 % to ±4 % depending on product and calibration.

- 🧠 Control & Electronics

- Control System: PLC-based control with HMI touchscreen for easy parameter setup.

- Sealing System: Pneumatic or servo actuated vertical & horizontal heat sealing.

- Temperature Control: PID digital temperature controllers for accurate sealing.

- Film Pulling & Registration: Servo/AC drive or belt pull system; photoelectric registration optional.

- Control System: Typically PLC or electronic controller for sealing and timing; basic machines may use simple logic controls.

- Sensors: Photoelectric eye/registration sensor for accurate pouch cut & placement (in many models).

### 📊 Filling & Product Range

- Filling Range (Weight):
• Typical ranges: 100g – 200g ,500 g – 1000 g  (interchangeable cups).

- Product Types: Free-flow items like sugar, salt, tea, pulses, coffee, spices, granules, small snacks, detergent powder, etc.

### 🕐 Performance

- Speed: ~18 – 28 pouches/min depending on product, pouch size, and operator control.

- Sealing Types: Center seal or three-side seal configurations.

### 📐 Dimensions & Weight

- Machine Footprint: Commonly around ~1150 mm x 1000 mm x 2200 mm (L×W×H) (varies by model).

- Net Weight: Approx 400 – 600 kg.

### ⚡ Power & Pneumatic

- Power Supply: Often 220 V AC, single phase, 50 Hz (some industrial models multi-phase Three Phace).

- Power Consumption: Around 2.5 kW typical (varies).

- Compressed Air: ~6 CFM @ ~6 bar (6 kg/cm²) for pneumatic operations.

### 🛠️ Construction

- Frame: Mild steel body with stainless steel (SS) product contact parts.

- Hopper & Cup Filling Components: Food-grade SS or compatible materials.

### 📦 Film & Pouch Details

- Film Material: Heat-sealable laminated film rolls.

- Film Width & Pouch Size: Typically customizable based on product — up to ~450 mm film width.

- 🧩 Optional Features

- Batch coding/printing unit

- Nitrogen flushing for extended shelf-life packaging

- Adjustable cup sets for varied fill weights

## 📌 Typical Applications

✔ Snacks & small granules (e.g., kurkure, puffs)
✔ Tea, coffee, sugar, spices & pulses
✔ Free-flow powders (e.g., detergents)
✔ Seeds, nuts & similar products requiring cup dosing

## 📍 How It Works (Quick Overview)

- Film Roll Unwinds: Film is fed from a roll and shaped into a pouch.

- Filling: Volumetric cup fills product; operator or pneumatic trigger releases product into pouch.

- Sealing: Pneumatic heat seals pouch (center or 3-side).

- Cut & Output: Completed pouch is cut and discharged.